Experiment No 10

BECS 32461

Digital Signal Processing Exercise

Student Name: W. K. G. K. Jayawardana

Student No: EC/2021/006

Date Performed: 2025/12/12

Date Submitted: 2025/12/12

Q01.a

fs = 500;

t = 0:1/fs:2;

noise = 2.5*randn(size(t));

x = sin(16*pi*t) + 0.7*sin(100*pi*t) + 0.5*sin(200*pi*t) + noise;

plot(t, x);

title('Sampled Signal x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

Q01.b

Yes. Choosen sampling frequency is satisfying Nyquist criteria for 𝑥(𝑡) = sin(16𝜋𝑡) + 0.7 sin(100𝜋𝑡) + 0.5 sin(200𝜋𝑡) + 𝑛(𝑡) because Nyquist rate of this signal is 200Hz and 500Hz(fs) is greater than that.

Q02.a

fs = 500;

t = 0:1/fs:2;

noise = 2.5*randn(size(t));

x = sin(16*pi*t) + 0.7*sin(100*pi*t) + 0.5*sin(200*pi*t) + noise;

N = length(x);

Y = fft(x);

f = (0:N-1)*(fs/N);

plot(f,abs(Y));

title('Magnitude Spectrum of x[n]');

xlabel('Frequency (Hz)');

ylabel('|fft(x)|');

xlim([0 250]);

grid on;

Q02.b

8Hz, 50Hz, 100Hz likely belongs to the true cosmic wave.

Q03.

N = 80;

fp = [0.008 0.08];

fs = 500;

x_fir = fir1(N, fp, 'bandpass', hamming(N+1));

figure;

[h_ham, w_ham] = freqz(x_fir, 1, 1024, fs);

plot(w_ham*fs/(2*pi), 20*log10(abs(h_ham)), 'r-'); hold on;

xlabel('Frequency(Hz)');

ylabel('Magnitude');

title('Magnitude Response');

grid on;

Hamming gives low ripple and good side-lobe suppression.

Q04.

Q04.a

Q04.b

N = 80;

fp = [0.008 0.08];

fs = 500;

a = 1;

x_fir = fir1(N, fp, 'bandpass', hamming(N+1));

figure;

[h_ham, w_ham] = freqz(x_fir, 1, 1024, fs);

[h_fda, w_fda] = freqz(b, a, 1024, fs);

plot(w_ham*fs/(2*pi), 20*log10(abs(h_ham)), 'r-'); hold on;

plot(w_fda*fs/(2*pi), 20*log10(abs(h_fda)), 'b-');

xlabel('Frequency(Hz)');

ylabel('Magnitude(db)');

title('Magnitude Response');

grid on;

legend('Hamming FIR','FDA Tool');

Q04.c

- FDA tool signal sharper than previously designed filter.

- So FDA tool is more suitable for preserving the cosmic message ehile removing interference.

Q05.

fs = 500;

t = 0:1/fs:2;

a = 1;

figure;

subplot(2,1,1);

plot(t, x);

title('Sampled Signal x(t)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(2,1,2);

y_fda = filter(b, a, x);

plot(t, y_fda);

title('Filtered Signal y[n] (FDA Tool FIR)');

xlabel('Time(s)');

ylabel('Amplitude');

Q06.

fs = 500;

t = 0:1/fs:2;

X = fft(x);

mask = zeros(size(X));

for k = 1:length(f)

if (f(k) >= 2 && f(k) <= 20)

mask(k) = 1;

mask(end-k+1) = 1;

end

end

X_masked = X .* mask;

x_rec = real(ifft(X_masked));

figure;

plot(t, x_rec, 'LineWidth', 1.2); hold on;

plot(t, y_fda, 'LineWidth', 1.2);

legend('IFFT Reconstructed','FDA Tool FIR Output');

title('Time-domain Comparison');

xlabel('Time (s)');

Q07.a

I choose Elliptic because it is more stable one.

Fs = 500;

Wp = [2 20] / (Fs/2);

Ws = [1 25] / (Fs/2);

Rp = 1;

Rs = 40;

[biir, aiir] = ellip(4, Rp, Rs, Wp, 'bandpass');

figure;

freqz(biir, aiir, 1024, Fs);

title('IIR Elliptic Bandpass Filter Response');

y_iir = filter(biir, aiir, x);

figure;

plot(t(1:500), x(1:500), 'LineWidth', 1); hold on;

plot(t(1:500), y_iir(1:500), 'LineWidth', 1.2);

legend('Original x[n]','IIR Filtered');

title('IIR vs Noisy');

xlabel('Time (s)');

figure;

zplane(biir, aiir);

title(' Pole-Zero Plot of Elliptic IIR Filter');

Q07.d

Q07.e

Q07.f

Q07.g

- IIR has sharper transition but FIR is more stable (all poles at 0).

- FIR FDA Tool filter better for distortion-free cosmic signal recovery.