Experiment No 08

BECS 32461

Paper E

INFINITE IMPULSE RESPONSE (IIR) FILTER DESIGN

Student Name: W. K. G. K. Jayawardana

Student No: EC/2021/006

Date Performed: 2025/11/14

Date Submitted: 2025/11/14

PROCEDURE

F01.

alphap = 3;

alphas = 15;

fp = 500;

fs = 750;

f = 2000;

% Normalize the frequencies

omp = 2 * fp / f;

oms = 2 * fs / f;

% Determine the order and cutoff frequency

[N, Wn] = buttord(omp, oms, alphap, alphas);

% Display the order and cutoff frequency

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Butterworth low-pass filter

[b, a] = butter(N, Wn, 'low');

% Display the filter coefficients

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi; % Frequency ranges from 0 to π (normalized)

[h, om] = freqz(b, a, w, 'whole');% The frequency response

m = abs(h);

% Magnitude of the frequency response

an = angle(h); % Phase of the frequency response

[bz,az] = bilinear(b,a,f);

figure;

subplot(2,1,1);

plot(w/pi,mag2db(m),'-b','LineWidth',1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Butterworth low-pass Filter Frequency Response');

grid on;

subplot(2,1,2);

plot(w/pi,an,'-b','LineWidth',1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Butterworth low-pass Filter Phase Response');

grid on;

F02.

alphap = 3;

alphas = 15;

fp = 500;

fs = 750;

f = 2000;

% Normalize the frequencies

omp = 2 * fp / f;

oms = 2 * fs / f;

% Determine the order and cutoff frequency

[N, Wn] = buttord(omp, oms, alphap, alphas);

% Display the order and cutoff frequency

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Butterworth high-pass filter

[b, a] = butter(N, Wn, 'high');

% Display the filter coefficients

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi; % Frequency ranges from 0 to π (normalized)

[h, om] = freqz(b, a, w, 'whole');% The frequency response

m = abs(h);

% Magnitude of the frequency response

an = angle(h); % Phase of the frequency response

[bz,az] = bilinear(b,a,f);

figure;

subplot(2,1,1);

plot(w/pi,mag2db(m),'-b','LineWidth',1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Butterworth low-pass Filter Frequency Response');\

grid on;

subplot(2,1,2);

plot(w/pi,an,'-b','LineWidth',1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Butterworth low-pass Filter Phase Response');

grid on;

EXERCISE

E01.

alphap = 2.5;

alphas = 40;

fp = 500;

fs = 900;

f = 3000;

% Normalize

omp = 2 * fp / f;

oms = 2 * fs / f;

% Order & cutoff

[N, Wn] = cheb1ord(omp, oms, alphap, alphas);

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Chebyshev Type I low-pass

[b, a] = cheby1(N, alphap, Wn, 'low');

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

an = angle(h);

[bz, az] = bilinear(b, a, f);

figure;

subplot(2,1,1);

plot(w/pi, mag2db(m), '-b', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Chebyshev Type I Low-pass Filter Frequency Response');

grid on;

subplot(2,1,2);

plot(w/pi, an, '-b', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Chebyshev Type I Low-pass Filter Phase Response');

grid on;

alphap = 2.5;

alphas = 40;

fp = 500;

fs = 900;

f = 3000;

% Normalize

omp = 2 * fp / f;

oms = 2 * fs / f;

% Order & cutoff

[N, Wn] = cheb1ord(omp, oms, alphap, alphas);

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Chebyshev Type I high-pass

[b, a] = cheby1(N, alphap, Wn, 'high');

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

an = angle(h);

[bz, az] = bilinear(b, a, f);

figure;

subplot(2,1,1);

plot(w/pi, mag2db(m), '-b', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Chebyshev Type I High-pass Filter Frequency Response');

grid on;

subplot(2,1,2);

plot(w/pi, an, '-b', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Chebyshev Type I High-pass Filter Phase Response');

grid on;

E02.

alphas = 25;

alphap = 1.2;

fp = 600;

fs = 1000;

f = 5000;

% Normalize

omp = 2 * fp / f;

oms = 2 * fs / f;

% Order & cutoff

[N, Wn] = cheb2ord(omp, oms, alphap, alphas);

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Chebyshev Type II low-pass

[b, a] = cheby2(N, alphas, Wn, 'low');

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

an = angle(h);

[bz, az] = bilinear(b, a, f);

figure;

subplot(2,1,1);

plot(w/pi, mag2db(m), '-y', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Chebyshev Type II Low-pass Filter Frequency Response');

grid on;

subplot(2,1,2);

plot(w/pi, an, '-b', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Chebyshev Type II Low-pass Filter Phase Response');

grid on;

alphas = 25;

alphap = 1.2;

fp = 600;

fs = 1000;

f = 5000;

% Normalize

omp = 2 * fp / f;

oms = 2 * fs / f;

% Order & cutoff

[N, Wn] = cheb2ord(omp, oms, alphap, alphas);

disp('Order of the filter n =');

disp(N);

disp('Cutoff frequency Wn = ');

disp(Wn);

% Design Chebyshev Type II high-pass

[b, a] = cheby2(N, alphas, Wn, 'high');

disp('Filter coefficients b =');

disp(b);

disp('Filter coefficients a =');

disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

an = angle(h);

[bz, az] = bilinear(b, a, f);

figure;

subplot(2,1,1);

plot(w/pi, mag2db(m), '-g', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Chebyshev Type II High-pass Filter Frequency Response');

grid on;

subplot(2,1,2);

plot(w/pi, an, '-r', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Phase in Radians');

title('Chebyshev Type II High-pass Filter Phase Response');

grid on;

E03.

alphap = 1.5;

alphas = 45;

fp1 = 140;

fs1 = 148;

fs2 = 152;

fp2 = 160;

f = 1200;

omp = [2*fp1/f 2*fp2/f];

oms = [2*fs1/f 2*fs2/f];

[N, Wn] = buttord(omp, oms, alphap, alphas);

disp('Order of filter =');

disp(N);

disp('Cutoff frequencies = ');

disp(Wn);

[b, a] = butter(N, Wn, 'stop');

disp('Filter coefficients b ='); disp(b);

disp('Filter coefficients a ='); disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

an = angle(h);

[bz, az] = bilinear(b, a, f);

figure;

subplot(2,1,1);

plot(w/pi, m, 'y-.', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Magnitude');

title('Butterworth Band-stop Filter Response');

grid on;

subplot(2,1,2);

plot(w/pi, mag2db(m), 'b--', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('dB');

title('Butterworth Band-stop Filter Response (dB)');

grid on;

E04.

alphap = 1;

alphas = 40;

fp1 = 800;

fp2 = 1800;

fs1 = 500;

fs2 = 2500;

f = 8000;

omp = [2*fp1/f 2*fp2/f];

oms = [2*fs1/f 2*fs2/f];

[N, Wn] = buttord(omp, oms, alphap, alphas);

disp('Order of filter =');

disp(N);

disp('Cutoff frequencies = ');

disp(Wn);

[b, a] = butter(N, Wn, 'bandpass');

disp('Filter coefficients b ='); disp(b);

disp('Filter coefficients a ='); disp(a);

w = 0:0.01:pi;

[h, om] = freqz(b, a, w, 'whole');

m = abs(h);

[bz, az] = bilinear(b, a, f);

figure;

plot(w/pi, mag2db(m), 'y', 'LineWidth', 1);

xlabel('Normalized Frequency');

ylabel('Gain in dB');

title('Butterworth Band-pass Filter Response');

grid on;

% Generate sonar signal

t = 0:1/f:1;

x = sin(2*pi*300*t) + sin(2*pi*1200*t) + sin(2*pi*3000*t);

% Filter the signal

y = filter(b, a, x);

figure;

subplot(2,1,1);

plot(t, x, 'b');

xlabel('Time (s)');

ylabel('Amplitude');

title('Original Sonar Signal');

grid on;

subplot(2,1,2);

plot(t, y, 'y');

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered Sonar Signal');

grid on;