Experiment No 06

BECS 32461

Paper A

LINEAR AND CIRCULAR CONVOLUTION

Student Name: W. K. G. K. Jayawardana

Student No: EC/2021/006

Date Performed: 2025/10/31

Date Submitted: 2025/10/31

PROCEDURE

F01.

x = [1 2 3 4];

h = [1 2 1 2];

m = length(x);

n = length(h);

L = m + n - 1;

N = max(m, n);

if m < n

x = [x zeros(1, N - m)];

h = [h zeros(1, N - n)];

end

ylc_manual = zeros(1, length(x) + length(h) - 1);

for n = 1:length(ylc_manual)

for k = 1:length(x)

if (n - k + 1) > 0 && (n - k + 1) <= length(h)

ylc_manual(n) = ylc_manual(n) + x(k) * h(n - k + 1);

end

end

end

ycc_manual = zeros(1, N);

for k = 0:N-1

sum = 0;

for j = 0:N-1

sum = sum + x(mod(k - j, N) + 1) * h(j + 1);

end

ycc_manual(k + 1) = sum;

end

figure;

subplot(3, 1, 1);

stem(0:length(x)-1, x, 'b', 'DisplayName', 'x[n]');

hold on;

stem(0:length(x)-1, h, 'r', 'DisplayName', 'h[n]');

hold off;

title('Input Sequences (Manual)');

xlabel('Sample Index');

ylabel('Amplitude');

legend;

subplot(3, 1, 2);

stem(0:L-1, ylc_manual, 'g');

title('Manual Linear Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

subplot(3, 1, 3);

stem(0:N-1, ycc_manual, 'm');

title('Manual Circular Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

F02.

x = [1 2 3 4];

h = [1 2 1 2];

m = length(x);

n = length(h);

L = m + n - 1;

N = max(m, n);

x_padded = [x zeros(1, L - N)];

h_padded = [h zeros(1, L - N)];

ylc_fft = ifft(fft(x_padded) .* fft(h_padded));

ycc_fft = ifft(fft(x).* fft(h));

figure;

subplot(3, 1, 1);

stem(0:m-1, x, 'b', 'DisplayName', 'x[n]');

hold on;

stem(0:m-1, h, 'r', 'DisplayName', 'h[n]');

hold off;

title('Input Sequences (FFT-based)');

xlabel('Sample Index');

ylabel('Amplitude');

legend;

subplot(3, 1, 2);

stem(0:L-1, ylc_fft, 'g');

title('FFT-based Linear Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

subplot(3, 1, 3);

stem(0:N-1, ycc_fft, 'm');

title('FFT-based Circular Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

F03.

x = [1 2 3 4];

h = [1 2 1 2];

ylc_builtin = conv(x, h);

N = max(length(x), length(h));

ycc_builtin = cconv(x, h, N);

figure;

subplot(3, 1, 1);

stem(0:length(x)-1, x, 'b', 'DisplayName', 'x[n]');

hold on;

stem(0:length(h)-1, h, 'r', 'DisplayName', 'h[n]');

hold off;

title('Input Sequences (Built-In)');

xlabel('Sample Index');

ylabel('Amplitude');

legend;

subplot(3, 1, 2);

stem(0:length(ylc_builtin)-1, ylc_builtin, 'g');

title('Built-in Linear Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

subplot(3, 1, 3);

stem(0:N-1, ycc_builtin, 'm');

title('Built-in Circular Convolution');

xlabel('Sample Index');

ylabel('Amplitude');

EXERCISE

E01.

x = [1 0 -1];

h = [2 1];

N = max(length(x), length(h));

% --- Zero-padding for FFT method ---

x_padded_fft = [x, zeros(1, N - length(x))];

h_padded_fft = [h, zeros(1, N - length(h))];

% --- (a) Circular Convolution using FFT ---

y_fft = ifft(fft(x_padded_fft) .* fft(h_padded_fft));

% --- Built-in circular convolution (no manual padding) ---

y_builtin_no_padding = cconv(x, h, N);

% --- (c) Built-in circular convolution with manual zero-padding ---

h_padded = [h, zeros(1, length(x) - length(h))];

y_builtin_padded = cconv(x, h_padded, length(x));

% --- Display numerical results ---

disp('Circular Convolution using FFT:');

disp(y_fft);

disp('Built-in Circular Convolution (without manual padding):');

disp(y_builtin_no_padding);

disp('Built-in Circular Convolution with Manual Zero-Padding:');

disp(y_builtin_padded);

% --- (b) Plot all results using subplots ---

figure;

% Subplot 1: FFT Method

subplot(3,1,1);

stem(0:N-1, y_fft, 'r', 'filled');

title('Circular Convolution using FFT');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

% Subplot 2: Built-in cconv (No Padding)

subplot(3,1,2);

stem(0:N-1, y_builtin_no_padding, 'r', 'filled');

title('Built-in Circular Convolution (No Padding)');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

% Subplot 3: Built-in cconv (With Manual Zero-Padding)

subplot(3,1,3);

stem(0:length(y_builtin_padded)-1, y_builtin_padded, 'r', 'filled');

title('Built-in Circular Convolution (With Manual Zero-Padding)');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

sgtitle('Circular Convolution Comparison (All Methods)');

Circular convolution treats signals as if they are repeating in a circle. For this to work correctly, both signals need to be the same length so they can "wrap around" and align properly.

If one signal is shorter than the other, we add zeros to the end of its process called zero-padding. This makes both signals equal in length and ensures the calculation is accurate. Without padding, the result would be incorrect because the signals wouldn't align properly.

E02.

% --- (a) ---

x = [1 2 3];

h = [1 1 1 1];

m = length(x);

n = length(h);

L = m + n - 1; % Output length for linear convolution

% (i) Manual Convolution without Zero Padding (Correct Implementation)

ylc_manual_no_pad = zeros(1, L);

for i = 1:L

for k = 1:m

if (i - k + 1) > 0 && (i - k + 1) <= n

ylc_manual_no_pad(i) = ylc_manual_no_pad(i) + x(k) * h(i - k + 1);

end

end

end

% (ii) Manual Convolution with Zero Padding

x_padded = [x, zeros(1, L - m)];

h_padded = [h, zeros(1, L - n)];

ylc_manual_padded = zeros(1, L);

for i = 1:L

for k = 1:i

ylc_manual_padded(i) = ylc_manual_padded(i) + x_padded(k) * h_padded(i - k + 1);

end

end

% (iii) Built-in Convolution

ylc_builtin = conv(x, h);

disp('Manual Convolution (No Padding):');

disp(ylc_manual_no_pad);

disp('Manual Convolution (With Padding):');

disp(ylc_manual_padded);

disp('Built-in conv Function:');

disp(ylc_builtin);

% --- (b) Plotting the results ---

figure;

subplot(3,1,1);

stem(0:L-1, ylc_manual_no_pad, 'b', 'filled');

title('Manual Convolution (No Padding)');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

subplot(3,1,2);

stem(0:L-1, ylc_manual_padded, 'b', 'filled');

title('Manual Convolution (With Zero Padding)');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

subplot(3,1,3);

stem(0:L-1, ylc_builtin, 'b', 'filled');

title('Built-in conv Function');

xlabel('Sample Index (n)');

ylabel('Amplitude');

grid on;

sgtitle('Linear Convolution Comparison');

When calculating linear convolution, all the methods in the exercise give the same correct result. However, zero-padding is extremely important when you use the FFT (Fast Fourier Transform) method to find the linear convolution.

Here's why: using the FFT to multiply signals is a shortcut that naturally produces a circular convolution result. To get the correct linear convolution result, you must first pad both signals with zeros to a combined length of at least $M + N - 1$ (where M and N are the original lengths). This padding prevents the "wrap-around" effect of circular convolution from corrupting the answer.

E03.

% --- (i) ---

fs = 1000;

f = 5;

t = 0:1/fs:1-1/fs;

sine_wave = sin(2 * pi * f * t);

noise = 0.5 * randn(size(t));

noisy_signal = sine_wave + noise;

% --- (ii)  ---

h_avg = [0.25 0.25 0.25 0.25];

% --- (iii)---

filtered_signal = conv(noisy_signal, h_avg, 'same');

% --- (iv)---

figure;

subplot(3, 1, 1);

plot(t, sine_wave, 'k');

title('1. Original Clean Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

ylim([-2, 2]);

subplot(3, 1, 2);

plot(t, noisy_signal, 'b');

title('Signal with Added Noise');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

ylim([-2, 2]);

subplot(3, 1, 3);

plot(t, filtered_signal, 'r', 'LineWidth', 1.5);

title('Filtered Signal (Noise Reduced)');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

ylim([-2, 2]); n

A moving average filter smooths out a signal by reducing sharp spikes. It works by making each new point in the signal an average of the last few points from the original noisy signal.

Random noise is made of quick, random ups and downs. When you average a small group of these noisy points, the random positive and negative values tend to cancel each other out. This process removes the sharp, jerky parts of the noise while keeping the main, underlying signal (like the sine wave) intact. The result is a much cleaner and smoother signal.

E04.

% --- (i) ---

try

img_orig = imread('cameraman.tif');

catch

disp('Error: Could not find "cameraman.tif".');

disp('Please ensure the Image Processing Toolbox is installed and the file is accessible.');

return;

end

img_double = double(img_orig);

% --- (ii) ---

h_avg = (1/9) * ones(3, 3);

% Sharpening filter

h_sharp = [ 0 -1  0;

-1  5 -1;

0 -1  0];

% --- (iii) Perform 2D Convolution ---

img_blurred = conv2(img_double, h_avg, 'same');

% Apply sharpening filter

img_sharpened = conv2(img_double, h_sharp, 'same');

% --- (iv) ---

img_blurred = uint8(img_blurred);

img_sharpened = uint8(img_sharpened);

figure;

% Original Image

subplot(1, 3, 1);

imshow(img_orig);

title('Original Image');

% Blurred Image

subplot(1, 3, 2);

imshow(img_blurred);

title('Filtered (Averaging)');

% Sharpened Image

subplot(1, 3, 3);

imshow(img_sharpened);

title('Filtered (Sharpening)');